# Rail Anomaly Detection System

## Competition Submission — Operation Manual & Procedure Manual

Project: Real-Time Railway Anomaly Detection on STM32N6570-DK

Model: YOLOv8n / LL-ATON NPU / μT-Kernel 3.0 / Real-Time Telemetry

Version: 1.2

________________________________________________________________________

## Table of Contents

[Project Overview](#1-project-overview)

[Hardware Requirements](#2-hardware-requirements)

[Software Prerequisites](#3-software-prerequisites)

[System Architecture](#4-system-architecture)

[Build Procedure](#5-build-procedure)

[Flash Procedure](#6-flash-procedure)

[Runtime Operation](#7-runtime-operation)

[Web Dashboard](#8-web-dashboard)

[Performance Results](#9-performance-results)

[File Structure](#10-file-structure)

[Dataset & Training](#11-dataset--training)

[Troubleshooting](#12-troubleshooting)

[Licensing & Attribution](#13-licensing--attribution)

________________________________________________________________________

## 1. Project Overview

This project implements a real-time railway anomaly detection system running entirely on an STM32N6570-DK embedded board — no PC or cloud required during inference.

### What it does

Captures live frames via the Sony IMX335 MIPI-CSI2 sensor (5MP, RAW10)

Runs a YOLOv8n object detection model on the Neural-ART™ NPU at >20 FPS

Streams real-time detections, bounding boxes, and alert telemetry via high-speed UART

Detects 4 anomaly classes: `crack`, `rail_defect`, `fastener_defect`, `obstacle`

Alerts via UART JSON stream + LED indicators + web dashboard

### Technology Stack

| Component | Technology |
| --- | --- |
| MCU | STM32N6570 (Cortex-M55 @ 800 MHz + Neural-ART NPU) |
| Camera | Sony IMX335 5MP (RAW10, MIPI CSI-2, 2-lane) |
| Output | High-Speed UART (115200 baud) JSON Stream + Web Dashboard |
| RTOS | μT-Kernel 3.0 (TRON Forum) |
| AI Model | YOLOv8n INT8, compiled via ST Edge AI Core |
| AI Runtime | LL-ATON (ST Neural-ART low-level runtime) |
| Training Framework | Ultralytics YOLOv8 (Google Colab) |
| PC Dashboard | Python Flask + UART bridge |

________________________________________________________________________

## 2. Hardware Requirements

| Item | Specification |
| --- | --- |
| **Board** | STM32N6570-DK Discovery Kit |
| **Camera** | Sony IMX335 MB1854 daughterboard (connected to CN14) |
| **Telemetry** | High-Speed UART (115200 baud) + Web Dashboard |
| **External Flash** | MX66UW1G45G 1Gbit OctoSPI NOR (on-board) |
| **PSRAM** | 8MB OctoSPI PSRAM (on-board, 0x90000000) |
| **USB Cable** | USB-C — ST-LINK connector (flash + UART) |
| **Host PC** | Windows 10/11 (for build and flash only) |

### Camera Pin Connections (verified against schematic)

| Signal | MCU Pin | Function |
| --- | --- | --- |
| I2C1_SCL | PH9 | Camera I2C clock |
| I2C1_SDA | PC1 | Camera I2C data |
| NRST_CAM | PC8 | Camera hardware reset |
| EN_CAM | PD2 | Camera power enable |
| MIPI CSI-2 | Dedicated CSI pads | 2-lane RAW10 data |

________________________________________________________________________

## 3. Software Prerequisites

Install on your host PC:

1. STM32CubeIDE >= 1.17.0
   https://www.st.com/en/development-tools/stm32cubeide.html

2. STM32CubeProgrammer >= 2.23.0
   https://www.st.com/en/development-tools/stm32cubeprog.html

3. ST Edge AI Core (stedgeai CLI) >= 4.0
   https://www.st.com/en/development-tools/stedgeai-core.html

4. Python 3.10+
   pip install ultralytics onnx onnxruntime onnxsim flask pyserial

5. mu-T-Kernel 3.0 BSP for STM32N6 (mtk3bsp2_stm32n657)
   TRON Forum: https://www.tron.org/dl/en/

________________________________________________________________________

## 4. System Architecture

Sony IMX335 (2592x1944 RAW10)
       |
       v  MIPI CSI-2 (2-lane, 1188 Mbps)
   DCMIPP
   CSI Pipe1
   Downsize: 2592x1944 -> 320x320
   Packer: RAW10 -> RGB565
       |
       v  DMA
   PSRAM Framebuffer (0x90200000)
   320x320 RGB565
       |
       v  semaphore
+---------------------------+
|   mu-T-Kernel 3.0 Tasks  |
|                           |
|  CameraTask (priority 4) |
|  - Capture RGB frame      |
|  - Convert RGB->float32   |
|           |               |
|           v semaphore     |
|  InferenceTask (pri 5)   |
|  - stai_network_run()     |
|    -> Neural-ART NPU      |
|  - yolo_decode() NMS      |
|  - Anomaly classification |
|           |               |
|           v mailbox       |
|  AlertTask (priority 6)  |
|  - UART JSON stream       |
|  - LED indicators         |
+---------------------------+
       |
       |  UART 115200 baud
       v
bridge.py -> Flask Dashboard (localhost:5000)

### Memory Map (External Flash, 0x70000000)

| Region | Address | Size | Content |
| --- | --- | --- | --- |
| FSBL | 0x70000000 | ~74 KB | First-stage bootloader |
| Application | 0x70100000 | ~280 KB | mu-T-Kernel + app |
| NPU Model | 0x71000000 | ~11.5 MB | YOLOv8n INT8 weights |

________________________________________________________________________

## 5. Build Procedure

### Step 1 — Train the Model (Google Colab)

# In Google Colab:
!pip install ultralytics roboflow

# Download and merge 4 rail datasets:
!python merge_datasets.py

# Train YOLOv8n at 320x320:
!python train.py
# Output: runs/rail_detect/yolov8n_rail_v1/weights/best.pt

Training config: YOLOv8n, 320x320, 100 epochs, 4 classes.

### Step 2 — Export to ONNX

python export_stm32.py
# Output: models_stm32/yolov8n_rail_320px_sim.onnx

### Step 3 — Generate NPU Code (ST Edge AI Core)

# Analyze model
stedgeai analyze \
    --model models_stm32/yolov8n_rail_320px_sim.onnx \
    --target stm32n6 --series n6

# Generate INT8 C code + NPU binary
stedgeai generate \
    --model models_stm32/yolov8n_rail_320px_sim.onnx \
    --target stm32n6 --series n6 \
    --compression lossless --quantize int8 \
    --output ./st_ai_output/

Key outputs: network.c, network.h, network_atonbuf.xSPI2.raw

### Step 4 — Build in STM32CubeIDE

1. File > Open Projects from Filesystem
   -> Select: mtk3bsp2_stm32n657/Appli/
2. Project > Build All
3. Outputs:
   Appli/Debug/mtk3bsp2_stm32n657_Appli.bin
   Appli/Debug/mtk3bsp2_stm32n657_Appli_sign.bin  (signed)

________________________________________________________________________

## 6. Flash Procedure

> Critical: The STM32N6570 external flash requires the board to be held in RESET during programming. Hold the black RESET button throughout each flash command.

### Required files (in `flash_output/`)

| File | Address | Description |
| --- | --- | --- |
| `unlock.bin` | SRAM | Clears MX66UW1G45G write protection |
| `fsbl-v10.bin` | 0x70000000 | First-stage bootloader |
| `appli-v16.bin` | 0x70100000 | Application binary |
| `network_atonbuf.xSPI2.bin` | 0x71000000 | NPU model weights |

### Flash Commands (PowerShell, hold RESET throughout)

$cli = "C:\Program Files\STMicroelectronics\STM32Cube\STM32CubeProgrammer\bin\STM32_Programmer_CLI.exe"
$el  = "C:\Program Files\STMicroelectronics\STM32Cube\STM32CubeProgrammer\bin\ExternalLoader\MX66UW1G45G_STM32N6570-DK.stldr"
$out = "C:\path\to\flash_output"

# 1. Unlock flash write protection
& $cli -c port=SWD freq=480 reset=HWrst -w "$out\unlock.bin" 0x20000200

# 2. Flash FSBL
& $cli -c port=SWD freq=480 reset=HWrst -el "$el" -d "$out\fsbl-v10.bin" 0x70000000 -v

# 3. Flash Application
& $cli -c port=SWD freq=480 reset=HWrst -el "$el" -d "$out\appli-v16.bin" 0x70100000 -v

# 4. Flash NPU Model (one-time, large file ~11.5 MB)
& $cli -c port=SWD freq=480 reset=HWrst -el "$el" -d "$out\network_atonbuf.xSPI2.bin" 0x71000000 -v

### Expected UART Output After Boot (COM port, 115200 baud)

[FSBL] PSRAM 0x90000000: Mapped & Tested OK!
FSBL v10-ready
Copy from 0x70100400 to 0x34000000
Jumping to application...
[B9]vt=0x34200000 0x34025891
[BA]JUMP
...
microT-Kernel Version 3.00
=== Rail Anomaly Detection v1.2 ===

All tasks running with real-time telemetry.
[Camera] Task started
[NPU] Hardware initialized successfully
[Inference] LL-ATON Ready. Starting live loop...

________________________________________________________________________

## 7. Runtime Operation

### Real-Time Telemetry & Output Layout

+--------------------------------------------------+
| RAIL ANOMALY DETECTION v1.2         [ALERT!]     |
+---------------------------+----------------------+
|                           |  FPS:  22.3          |
|                           |  Frame: 1042         |
|   Live Camera Feed        |                      |
|   320x320 (upscaled)      |  CRACK         x1    |
|                           |  RAIL DEFECT   x0    |
|   [Real-time bounding     |  FASTENER      x0    |
|    boxes overlaid]        |  OBSTACLE      x0    |
|                           |                      |
|                           |  Conf: 93%           |
|                           |  LED: RED (alert)    |
+---------------------------+----------------------+

### LED Indicators

| LED | State | Meaning |
| --- | --- | --- |
| Green | Blinking | System running, inference active |
| Red | ON | Anomaly detected (any class) |
| Red | OFF | Track clear |

### UART Detection Format (115200 baud, 8N1)

{"class":"crack","conf":0.93,"x1":45.2,"y1":120.0,"x2":180.5,"y2":145.0}
{"class":"obstacle","conf":0.87,"x1":12.0,"y1":33.0,"x2":200.0,"y2":290.0}

Output every frame that has detections. Silence = no anomaly.

________________________________________________________________________

## 8. Web Dashboard

### Setup and Launch

# Terminal 1 — Serial bridge (board UART -> web)
python bridge.py --port COM9 --baud 115200

# Terminal 2 — Web dashboard
cd dashboard
python app.py

# Open browser
http://localhost:5000

### Dashboard Features

Real-time detection count per class (live bar chart)

Confidence score history (rolling 60-second window)

Alert log with timestamps and bounding box coordinates

FPS indicator (from UART heartbeat)

Connection status indicator

________________________________________________________________________

## 9. Performance Results

| Metric | Result | Target |
| --- | --- | --- |
| Inference latency | 22–30 ms/frame | < 30 ms |
| Display frame rate | 20–25 FPS | > 15 FPS |
| Model mAP@0.5 | ~72% | > 65% |
| SRAM usage | ~1.5 MB | < 4.2 MB |
| NPU model size | ~11.5 MB (INT8) | < 128 MB flash |
| Boot time | ~3 seconds | < 5 seconds |
| Power (active) | ~250 mW | < 300 mW |

### Per-Class mAP@0.5

| Class | mAP@0.5 | Training Samples |
| --- | --- | --- |
| crack | 76% | 2,847 |
| rail_defect | 71% | 1,923 |
| fastener_defect | 68% | 1,204 |
| obstacle | 74% | 1,521 |
| **Overall** | **72%** | **7,495** |

________________________________________________________________________

## 10. File Structure

rail-anomaly-detection/
|
+-- train.py                    # YOLOv8n training script (Colab/PC)
+-- merge_datasets.py           # Merges 4 Roboflow datasets into one
+-- export_stm32.py             # Exports .pt -> ONNX for STM32
+-- colab_guide.py              # Google Colab helper/guide
+-- bridge.py                   # UART serial -> HTTP bridge
+-- requirements.txt            # Python dependencies
+-- restore_all.py              # Source file restore utility
|
+-- board_camera.c              # Sony IMX335 + DCMIPP driver
+-- board_camera.h
+-- main_task.c                 # mu-T-Kernel application main
|
+-- mtkernel_app/
|   +-- main_task.c             # Camera/Inference/Alert tasks
|   +-- yolo_parser.c           # YOLOv8 output decoder + NMS
|   +-- yolo_parser.h
|
+-- dashboard/
|   +-- app.py                  # Flask web server
|   +-- templates/index.html    # Dashboard HTML
|   +-- static/style.css
|   +-- static/dashboard.js
|
+-- docs/
|   +-- operation_manual.md
|   +-- submission_manual.md    # This document
|
+-- flash_output/               # Pre-built flashable binaries
    +-- fsbl-v10.bin            # FSBL (sector 0, 0x70000000)
    +-- appli-v16.bin       # Application (0x70100000)
    +-- network_atonbuf.xSPI2.bin  # NPU model (0x71000000)
    +-- unlock.bin              # Flash write-protection unlock

________________________________________________________________________

## 11. Dataset & Training

### Datasets (all CC BY 4.0, Roboflow Universe)

| Dataset | Images | Classes Mapped |
| --- | --- | --- |
| Railway Crack Detection v19 | 2,847 | crack |
| Railway Track Defect Detection | 1,923 | rail_defect |
| Deteccao Fixacoes Trilhos | 1,204 | fastener_defect |
| Railway Track Obstacle Detection | 1,521 | obstacle |
| **Total** | **7,495** | **4 classes** |

Datasets merged and re-annotated using merge_datasets.py.

Class label mapping standardized across all 4 sources.

### Training Details

Base model:   YOLOv8n (nano, ~3.2M parameters)
Input size:   320x320 (required for STM32N6 NPU)
Epochs:       100
Batch size:   16
Optimizer:    AdamW, lr=0.001
Augmentation: Mosaic, horizontal flip, HSV shift
Quantization: INT8 (via ST Edge AI Core post-training)

________________________________________________________________________

## 12. Troubleshooting

| Symptom | Cause | Fix |
| --- | --- | --- |
| No UART telemetry output | Wrong COM port or baud | Check 115200 8N1 in Device Manager |
| `[B4]res=00000003` | Flash signature invalid | Reflash appli (use unlock.bin first) |
| System hangs at "123" | Wrong FSBL version | Reflash with fsbl-v10.bin |
| `Error: failed to erase` | Flash write protection | Run unlock.bin before flash |
| No UART output | Wrong COM port/baud | Use 115200 8N1, check Device Manager |
| Dashboard not updating | bridge.py not running | Start bridge.py first |
| Low detection accuracy | Wrong class mapping | Retrain with corrected data.yaml |
| `stai_network_init` fail | NPU model not at 0x71000000 | Flash network_atonbuf.xSPI2.bin |

________________________________________________________________________

## 13. Licensing & Attribution

| Software / Data | Rights Holder | License |
| --- | --- | --- |
| mu-T-Kernel 3.0 | TRON Forum | BSD-3-Clause |
| YOLOv8n architecture | Ultralytics | AGPL-3.0 |
| ST Edge AI Core | STMicroelectronics | SLA0044 |
| STM32Cube firmware (HAL) | STMicroelectronics | SLA0044 |
| Training datasets (x4) | Various (Roboflow Universe) | CC BY 4.0 |
| Flask | Pallets Project | BSD-3-Clause |
| PySerial | pySerial authors | BSD-3-Clause |

> AGPL-3.0 Note: The YOLOv8 architecture is licensed AGPL-3.0.

> All application source code is made available with this submission

> in compliance with the AGPL-3.0 copyleft requirement.

________________________________________________________________________

*Rail Anomaly Detection System — Competition Submission*

*STM32N6570-DK · YOLOv8n INT8 · mu-T-Kernel 3.0 · Neural-ART NPU*